- Using sim.log, print the line numbers of all ERROR messages.

- Count the number of warnings in sim.log

- Case-insensitively search for the word setup violation in timing_report.rpt.

- Write a shell script to check if netlist.v contains the cell NAND2_X1. If found, print "Cell Found", else "Cell Not Found".

- From timing_report.rpt, print each line with the word violation and the 2 lines before it.

- From sim.log, print each ERROR along with the next 3 lines.Show both the line before and after fatal errors in sim.log.

- Search for both error and warning in sim.log with a single command.

- Use Search cells.txt contents to search from the netlist.v

- List all Verilog files in the folder that contain the keyword endmodule.

- From alu.v, extract all lines that start with the word always.

- From truth_table.txt, match the word nand only (not parts like nand2).

- From constraints.sdc, remove comments (lines starting with #).

- From sim_results.log, print all lines containing numbers.

- From netlist.v, identify any line with special characters (non-alphanumeric).

- Write a script that checks if the file timing_report.rpt contains the word slack.

If found, print "Slack Report Available".

Else, print "No Slack Data".

- Search for the keyword clk in both netlist.v and alu.v together. Show the filename and matching line.

- Run grep -s "missing" on stdcells.lib. Observe that it suppresses error messages if the pattern doesn’t exist.

- print the filename along with the line containing the word module.

- Use grep -h "pin" in stdcells.lib to print only matches without filenames.

- count number of error messages.

- Extract all input port definitions from alu.v using grep "input" and redirect them into a new file inputs.txt.

- From alu.v, find lines containing add_r OR xor_r OR and_r.

- In netlist.v, find all gates starting with N or I.

- In sim_results.log, print only lines ending with clk.

- Extract only module declarations (module ...) from all .v files.

- From truth_table.txt, extract only rows (skip comments starting with #).

- From sim_results.log, find any line where a or b = single digit number.

- In constraints.sdc, print only lines that define delay constraints (set_input_delay or set_output_delay).

- From alu.v, extract all always blocks (both combinational and sequential).

- Write a shell script that:

- Prompts the user for a cell name.

- Searches stdcells.lib for it.

- Prints "Available" if found, "Not Available" otherwise.

- Write a script that checks sim.log for fatal errors. If found, exit with code 1. If no fatal error, exit with 0.

- Create a list of all Verilog modules present in netlist.v and alu.v by grepping for module and storing results in modules.txt.

- Find all ports (input/output) defined in alu.v and netlist.v.

- Explore : Extract all standard cell names (NAND2_X1, INV_X1, DFF_X2) from netlist.v using grep with word boundaries (\< \>).

“We generate fears while we sit. We overcome them by action. Fear is nature’s way of warning us to get busy.” –  Dr. Henry Link